SDK Diag Shell

Switch Product

Apollo Coding Guide

(CONFIDENTIAL: Development Partners Only)

Rev. 2.0.0

02 August 2011

|  | Realtek Semiconductor Corp. No. 2, Innovation Road II, Hsinchu Science Park, Hsinchu 300, Taiwan Tel.: +886-3-578-0211 Fax: +886-3-577-6047 www.realtek.com |
| --- | --- |

COPYRIGHT

©2009-2010 Realtek Semiconductor Corp. All rights reserved. No part of this document may be reproduced, transmitted, transcribed, stored in a retrieval system, or translated into any language in any form or by any means without the written permission of Realtek Semiconductor Corp.

TRADEMARKS

Realtek is a trademark of Realtek Semiconductor Corporation. Other names mentioned in this document are trademarks/registered trademarks of their respective owners.

DISCLAIMER

Realtek provides this document “as is”, without warranty of any kind, neither expressed nor implied, including, but not limited to, the particular purpose. Realtek may make improvements and/or changes in this document or in the product described in this document at any time. This document could include technical inaccuracies or typographical errors.

USING THIS DOCUMENT

This document is intended for use by the system engineer when integrating with Realtek Switch Software SDK. Though every effort has been made to assure that this document is current and accurate, more information may have become available subsequent to the production of this guide. In that event, please contact your Realtek representative for additional information that may help in the development process.

REVISION HISTORY

| Revision | Release Date | Summary |
| --- | --- | --- |
| 0.0.1 | 2011/07/06 | Branch from management switch. |

Table of Contents

Chapter 1	 Introduction	4

Chapter 2	 Command Grammar	5

2.1	Conventions	5

2.2	Argument Types	5

2.3	CLI Parser Rules	5

Chapter 3	 Coding Procedure	8

Chapter 4	 Cparser Know Issue	9

# Chapter 1		Introduction

This document provides a summary of the commands supported on the REALTEK SDK diag shell Command Line Interface (CLI). The document is divided into two sections. Section 1 describes the convention, argument types, privileges and command modes, command help, and command line editing. Section 2 gives a brief usage and description of all commands included in the diag shell.

# Chapter 2		Command Grammar

## Conventions

The following conventions are used in the command syntax throughout this document:

- “|” indicate separate alternative, mutually exclusive arguments.

- “{}” indicate optional elements. 目前{}裡面只能有一個token

- “()” indicate a required choice.

- “ [<>]” indicate required choices within optional elements.

- Bold words indicates commands and keywords.

- Italics indicate user variables, example: “<TYPE:var>“

## Argument Types

The following argument types are recognized by the CLI and are used in the command syntax throughout this document:

- MACADDR	  – MAC address.

Example: “00:3B:51:A9:CC:07”

- PORT_LIST – Port list, separated by “,” or “-”.

Example: “1-3,6,8”

- IPV4ADDR  – IPv4 address.

Example: “192.168.1.100”

- IPV6ADDR	 – IPv6 address.

Example: ”101:234:689:ACD:151:03B:1A9:C07”

- UINT		 – unsigned integer. 十進位or十六進位數字輸入

Example: “1234”, “0x1234”

- HEX		 	 – hexadecimal integer. 十六進位數字輸入

Example: “0x1234”

其實 UINT 與 HEX 都一樣都可以輸入“1234”, “0x1234”, parser 會轉成正確的數值 HEX

## CLI Parser Rules

1. comments:

請寫在該指令前, 用//代表comment開頭, 每行代表一token,

ex:

//test config

//token config

//aaa config

//bbb config

test token aaa bbb

就算不寫comment 也請在指令上一行一定要加上//

Apollo 禁止不寫 cmment

ex:

//

test token aaa bbb ccc

2. ( aaa | bbb | ccc)

表示從aaa, bbb, ccc 中選一

ex: test token ( aaa | bbb | ccc ) eee

限制: 目前不能support  ( aaa bbb | ccc) 的形式

Solution: 改成 2 道command



support  aaa bbb

support  ccc

{ aaa }

表示 aaa 可有可無

ex: test  { <STRING:ccc> } bbb

限制: 目前{}裡面只能有一個token

若要實做以下command 請改成 2 道command

test  { key <STRING: key_string > } bbb



test  bbb

test  key <STRING:key_string> bbb

目前parser會用空白切token 所以指令要寫成 一定要加 空白“space”

test token ( aaa | bbb ) 不能是 test token (aaa|bbb)

3. <TYPE:var> --- var 請用小寫

指令格式:

1. major-feature + action + sub-feature + interface + ...
   ex: dot1x set port-based port-list <enable | disable>

   "action" 目前只包含 get, set, reset, del, add, create, destroy, save, dump
                 -- "get"  for 一筆資料欄位
                 -- "dump" for一次print所有資料 不會有分頁的功能

2. get顯示資料訊息時, 用 " : " (冒號前後各只有一個空白)來帶出資料訊息
   ex: dot1x get port-based 1            <== 輸入的指令
       port 1 port-based dot1x : enable  <== 輸出的資訊

   "get" 顯示的資料訊息只詳細到interface, 之後的可以同時顯示出

3. port PORT-LIST, mac mac-address等, 可簡化成 PORT-LIST, mac-address
    ex: dot1x <add | del> auth-mac port port-id vid vid mac mac-address
         改寫成 dot1x <add | del> auth-mac port-id vid mac-address

若遇到簡化後,會跟其他指令衝突時,可保留前面的keyword,以利parser做辨別

[note] Apollo 的command 都要保留 前面的keyword
    ex:
    底下兩指令,若分別省略掉group跟port之後, 當user輸入qos set remarking dscp 1 xxx     xxx.....時, parser 無法分辨輸入的 "1"是代表 "group-id" or "port-id" .
qos set remarking dscp group group-id internal-priority drop-precedence dscp
qos set remarking dscp port port-id group group-id

4. 針對port的設定, 除了PORT-LIST外, 多加"all" 可以設定所有ports
    ex: dot1x set port-based PORT-LIST <auth | unauth>
    改寫成:  dot1x set port-based <PORT-LIST | all> <auth | unauth>

NOTE: ( <PORT_LIST:port> | all ) 裡面的all 不包含CPU port, 若需要設定CPU port 則由PORT_LIST去指定

5. Get的顯示格式:

RTK.0> pie get template 1

Template : 1 xxx yyy zzz

field   0 : dmac1

field  10 : vid-range

field 100 : ethertype

rate      : 100

“Template”--- 不縮排

“xxx yyy zzz” --- 要顯示的一些說明資訊

“field” --- 前面空1個’\t’

每個欄位的”:”對齊

# Chapter 3		Coding Procedure

Create xxx.cli file and then write commands and comments into it.

Add “#include "./xxx.cli" into diag_shell.cli

Execute “./mk_parser.py –D CONFIG_SDK_DRIVER_NIC diag_shell.cli” to generate cparser_tree.c and cparser_tree.h

Move cparser_tree.c to src\parser

Move cparser_tree.h to include\parser

Copy function name from cparser_tree.h to diag_xxx.c and then implement function body.

NOTE:

1). 為了讓產生的指令能依照英文字母順序排列, 請注意在.cli file裡面的指令先後順序要排列, 在diag_shell.cli的#include xxx.cli也要依component名稱順序排列

# Chapter 4		Cparser Know Issue

1. 當可有可無選項“{ }”後面接有參數輸入時”< : >”, cparser自動產生的	token參數會有問題.

Example:

pie set police index ( invalid | dlb | srtcm | trtcm ) { color-aware } <UINT:yellow-dp> <UINT:pir> <UINT:cir> <UINT:tp> <UINT:tc> <UINT:pbs> <UINT:cbs>

在auto generate的cparser_tree.c裏cparser_glue_pie_set_police_index_invalid_dlb_srtcm_trtcm_color_aware_yellow_dp_red_dp_pir_cir_tp_tc_pbs_cbs (cparser_t *parser)

{

…

rc = cparser_get_uint(&parser->tokens[5], &yellow_dp_val);

assert(CPARSER_OK == rc);

…

}

但當user有輸入color-aware選項時, yellow_dp_val應該拿token[6]

Solution: 實作時,將此指令拆成兩個指令

pie set police index ( invalid | dlb | srtcm | trtcm ) <UINT:yellow-dp> <UINT:red-dp> <UINT:pir> <UINT:cir> <UINT:tp> <UINT:tc> <UINT:pbs> <UINT:cbs>

pie set police index ( invalid | dlb | srtcm | trtcm ) color-aware <UINT:yellow-dp> <UINT:red-dp> <UINT:pir> <UINT:cir> <UINT:tp> <UINT:tc> <UINT:pbs> <UINT:cbs>

2. 當有兩個’多選一’連接在一起時, 若有另一指令的深度跟第一個’多選一’相同時, 其後的token串接時會有問題

Example: 下面兩指令, cparser串接後會有問題:

eee set ( <PORT_LIST:port> | all ) sleep-mode ( pause | queue-empty | rate-limit ) ( disable | enable )
eee set ( <PORT_LIST:port> | all ) sleep-mode rate-limit <UINT:RATE>

cparser會串接成

eee set ( <PORT_LIST:port> | all ) sleep-mode ( pause | queue-empty | rate-limit ) ( disable | enable | <UINT:RATE>)

Solution: 實作時, 對調兩指令的順序 (or 拆成多個指令)

eee set ( <PORT_LIST:port> | all ) sleep-mode rate-limit <UINT:RATE>

eee set ( <PORT_LIST:port> | all ) sleep-mode ( pause | queue-empty | rate-limit ) ( disable | enable )

Realtek Semiconductor Corp.

Headquarters

No. 2, Innovation Road II

Hsinchu Science Park, Hsinchu 300, Taiwan

Tel.: +886-3-578-0211. Fax: +886-3-577-6047

www.realtek.com